FIXATION AND EMBEDDING OF PLANT SAMPLES

- Fixative: 2.5% glutaraldehyde + 2.0% paraformaldehyde in cacodylate buffer 0.1 M (pH= 7.4).

- Cut leaves in small pieces (1x1 mm2). The samples must be cut in a big drop of the fixative solution.

- Place the samples into vials containing the fixative solution (the samples will float on the fixative).

- Apply a vacuum to the samples until they are at the bottom of the fixative. Start fixation time once the samples have sunk.

- Fixation of the samples: around 24 hours at 4C (the first 2-3 hours, fixation should be carried out at room temperature to avoid chilling injuries on the tissue).

- 3 x 20' washes in cacodylate buffer 0.1 M (pH= 7.4).

- Postfixation in 1% osmium tetroxide containing 0.1 M cacodylate buffer (pH= 7.4).

- 3 x 20' washes in cacodylate buffer 0.1 M (pH= 7.4).

- Dehydration in acetone series:

50% ........................ 20 minutes

70%..........................20 minutes

80% .........................20 minutes

90% .........................20 minutes

95% .........................20 minutes

100% ........................20 minutes

100% ........................20 minutes

100% ........................20 minutes

- Infiltration in Spurr resin:

Acetone:Spurr

3:1 ........................... overnight

2:2 ........................... 3-4 hours.

1:3 ........................... overnight

Resin ........................ 5 hours

Resin ........................ 2 hours

Resin ....................... overnight

- Embedding of the samples in silicone molds. Polymerization for 24 hours at 60 C.