Nikon Start-Up Procedure

1.  Turn the A1 Controller (#1) to the ON position by pushing in the power button, located on the top, left side of the lower control unit.

2.  Turn the Laser Power Key (#2) to the ON position.  This will turn on all of the laser lines.  The lasers are ready to use when the lights stop flashing.

3.  Turn the Stage Controller (#3) to the ON (“I”) position.

This box is located below the air table.

4.  Turn the Automated Filter Wheel (#4) to the ON position.

5.  Turn the X-Cite LED power supply (#5) to the ON (“I”) position.

This box is located below the air table.

6.  Turn the microscope (#6) to the ON (“I”) position.

This button is located on the right of the microscope, near the very back.

7.  Turn the computer (#7) and the monitor ON.

8.  Double click on the NIS-Elements Acquisition & Analysis icon.

Select “Nikon Confocal”.

Nikon Shut-Down Procedure

1.  Exit the NIS-Elements software and shut-down the Windows software.

2.  Turn the microscope (#6) to the OFF position.

3.  Turn the X-Cite (#5) to the OFF (“O”) position.

4.  Turn the Automated Filter Wheel (#4) to the OFF position.

5.  Turn the Stage Controller (#3) to the OFF (“O”) position.

6.  Turn the Laser Power key (#2) to the OFF (“O”) position.

7.  Turn the A1 Controller (#1) to the OFF position by pushing in the power button, located on the top, left side of the lower control unit.