Nikon C2 Start-Up Procedure

1.  Turn the Microscope (#1) to the ON (“I”) position.

2.  Turn the Z-Drive Controller (#2) to the ON (“I”) position.  This box is located on the air table, to the left of the microscope.

3.  Turn the Laser Unit (#3) to the ON (“I”) position.  This box is located behind the monitor.  The power button is in the back of the unit, lower right.

4.  Turn the Laser Power Key (#4) to the ON position.  This will turn on all of the laser lines.

5.  Turn the Fluorescence LED power supply (#5) to the ON (“I”) position.

This box is located below the table.

6.  Turn the Confocal Controller (#6) ON.  This box is located below the table.

7.  Turn the computer (#7) and the monitor ON.

8.  Double click on the NIS-Elements Acquisition & Analysis icon.

Select “Nikon Confocal”.

Nikon Shut-Down Procedure

1.  Exit the NIS-Elements software and shut-down the Windows software.

2.  Turn the Laser Power key (#4) to the OFF position.

3.  Turn the Laser Unit (#3) to the OFF (“O”) position.

4.  Turn the Z-Drive Controller (#2) to the OFF (“O”) position.

5.  Turn the microscope (#1) to the OFF (“O”) position.

6.  Turn the Fluorescence LED (#5) to the OFF (“O”) position.

5.  Turn the Confocal Controller (#6) OFF.