Nikon AXR Start-Up Procedure

- Push Button #1 to turn the Laser Power ON (“I”) position.

The power button is located on the left side of the middle control unit.

- Push Button # 2 to turn the AXR Confocal Controller ON (“I”) position.

The power button is located on the left side of the lower control unit.

- Turn the Laser Power Key (#3) to the ON position.

This will turn on the 405, 488, 561, and 647 laser lines. To turn on the 730nm laser line, turn the 730 Laser Power Key to the ON position.

- Push Button #4 to turn the AXR Controller ON (blue light).

- Turn the Automated XY Stage Controller (#5) to the ON (“I”) position.

This box is located below the air table on the left.

- Turn the Piezo Stage Controller (#6) to the ON (“I”) position.

This box is located below the air table in the middle.

- Turn the Automated Microscope Controller (#7) to the ON (“I”) position.

This box is located below the air table on the right.

- Turn the LED power supply (#8) to the ON (“I”) position.

This box is located behind the microscope, on the top, back position.

- Turn the Computer (#9) and the monitor ON.

- Double click on the NIS-Elements AR icon.

Select “Nikon AX Acquisition Driver”.

Nikon AXR Shut-Down Procedure

- Exit the NIS-Elements software and shut down the Windows software.

- Turn the LED power supply (#8) to the OFF (“O”) position.

- Turn the Automated Microscope Controller (#7) to the OFF (“O”) position.

- Turn the Piezo Stage Controller (#6) to the OFF (“O”) position.

- Turn the Automated XY Stage Controller (#5) to the OFF (“O”) position.

- Push Button #4 to turn the AXR Controller OFF.

- Turn the Laser Power key (#3) to the OFF position.

- Turn the AXR Confocal Controller button (#2) to the OFF (“O”) position.

- Turn the Laser Power button (#1) to the OFF (“O”) position.